# BCIT Make+ Private Documentation

## Unitree Go2W Agent SDK - Complete Codebase Reference

Author: Christopher Chong

Organization: BCIT Make+

Purpose: Assistive Technology Development

Last Updated: January 2025

__________________________________________________

## Table of Contents

Project Overview

Maestro Integration

Languages Used

Directory Structure

Core System Components

ROS1 Components (unitree_ros1/)

ROS2 Components (unitree_ros2/)

Configuration Files

Key Python Files Reference

ROS Topics & Services

Hardware Interfaces

Build & Run Commands

Extension Points for Assistive Tech

__________________________________________________

## Project Overview

This SDK provides a unified control system for the Unitree Go2W quadruped robot (wheeled variant) with an AgileX Piper 6-DOF robotic arm. The system bridges ROS1 (Noetic) and ROS2 (Foxy) ecosystems to provide:

Locomotion Control: Velocity-based movement of the robot dog

Robotic Arm Control: 6 joint angles + gripper via CAN bus

SLAM & Navigation: LiDAR-based mapping and autonomous navigation

Object Detection: YOLOv8/v9 with 3D localization

Sensor Integration: RealSense RGB-D camera, Hesai LiDAR, IMU

### System Architecture (with Maestro)

┌─────────────────────────────────────────────────────────────────┐

│                         MAESTRO                                  │

│            VLM-Based Coding Agent (Policy Planner)               │

│    ┌─────────────────────────────────────────────────────────┐  │

│    │  Vision-Language Model (Qwen2.5-VL / Claude / GPT)      │  │

│    │  - Interprets natural language instructions             │  │

│    │  - Generates executable Python code                     │  │

│    │  - Closed-loop feedback from execution results          │  │

│    └─────────────────────────────────────────────────────────┘  │

│                              │                                   │

│    ┌─────────────────────────┼─────────────────────────────┐    │

│    │              Hierarchical Toolset                      │    │

│    ├──────────────┬──────────────┬──────────────┬──────────┤    │

│    │  Perception  │   Planning   │   Control    │ Geometry │    │

│    │  - YOLO      │   - Nav2     │   - Arm SDK  │ - tf2    │    │

│    │  - 3D Det    │   - Motion   │   - Dog SDK  │ - NumPy  │    │

│    │  - Keypoints │   - Explore  │   - Gripper  │ - PCL    │    │

│    └──────────────┴──────────────┴──────────────┴──────────┘    │

└─────────────────────────────────────────────────────────────────┘

│

Python API / ROS Topics & Services

│

┌───────────────────────────────┼───────────────────────────────┐

│                         ROS1 Bridge                            │

│                    (dynamic_bridge)                            │

└───────────────────────────────┼───────────────────────────────┘

│                                       │

┌───────┴───────┐                     ┌────────┴────────┐

│    ROS1       │                     │      ROS2       │

│   (Noetic)    │                     │     (Foxy)      │

├───────────────┤                     ├─────────────────┤

│ - Faster-LIO  │                     │ - Nav2          │

│ - HesaiLiDAR  │                     │ - YOLO          │

│ - SLAM        │                     │ - RealSense     │

└───────────────┘                     │ - Piper Arm     │

│ - Msg Converter │

└─────────────────┘

│

┌───────────────┼───────────────┐

│               │               │

┌─────┴─────┐   ┌─────┴─────┐   ┌─────┴─────┐

│  Go2 Dog  │   │  Piper    │   │  Sensors  │

│(Ethernet) │   │(CAN Bus)  │   │(USB/ETH)  │

└───────────┘   └───────────┘   └───────────┘

__________________________________________________

## Maestro Integration

Website: https://maestro-robot.github.io/

### What is Maestro?

Maestro is a VLM-based (Vision-Language Model) coding agent that generates programmatic policies for robot control. Instead of end-to-end trained models, Maestro uses a VLM to interpret natural language instructions and write executable Python code that composes specialized robotics modules.

Key Insight: Maestro was specifically designed for and tested on the Unitree Go2-W with AgileX PiPER arm - exactly this hardware platform!

### How Maestro Works

┌─────────────────────────────────────────────────────────────────┐

│                    MAESTRO EXECUTION LOOP                        │

│                                                                  │

│   1. USER INSTRUCTION                                            │

│      "Pick up the red cup and bring it to me"                   │

│                         │                                        │

│                         ▼                                        │

│   2. VLM INTERPRETS & GENERATES CODE                            │

│      ┌─────────────────────────────────────────────────────┐    │

│      │ # Maestro-generated policy                          │    │

│      │ cup = detect_object("red cup")                      │    │

│      │ position_3d = get_3d_position(cup)                  │    │

│      │ navigate_to(position_3d, offset=0.5)                │    │

│      │ arm_reach(position_3d)                              │    │

│      │ gripper_close()                                     │    │

│      │ navigate_to(user_position)                          │    │

│      │ gripper_open()                                      │    │

│      └─────────────────────────────────────────────────────┘    │

│                         │                                        │

│                         ▼                                        │

│   3. EXECUTE & OBSERVE                                          │

│      - Run code against ROS modules                             │

│      - Capture camera images + stdout                           │

│                         │                                        │

│                         ▼                                        │

│   4. FEEDBACK LOOP                                              │

│      - VLM analyzes execution results                           │

│      - Replans if needed (object moved, obstacle detected)      │

│      - Continues until task complete                            │

│                                                                  │

└─────────────────────────────────────────────────────────────────┘

### Maestro's Hierarchical Toolset

Maestro organizes robot capabilities into a carefully curated set of modules:

#### Perception Modules (Tiered: Fast → Precise)

| Tier | Module | Speed | This Codebase Component |
| --- | --- | --- | --- |
| 1 | Raw camera input | Fastest | `/camera/color/image_raw` |
| 2 | YOLO bounding boxes | Fast | `yolov8_node.py` → `/yolov8_node/detections` |
| 3 | Mask centroids | Medium | `yolov8_node.py` (segmentation masks) |
| 4 | VLM-selected keypoints | Precise | `detect_3d_node.py` → `/detections_3d` |

#### Planning Modules

| Module | Purpose | This Codebase Component |
| --- | --- | --- |
| Motion Planning | Collision-free paths | Nav2 + point cloud costmaps |
| Active Exploration | Information gathering | LiDAR-Inertial Odometry (Faster-LIO) |
| Semantic Mapping | Object tracking | YOLO + 3D projection |

#### Control Modules

| Module | Purpose | This Codebase Component |
| --- | --- | --- |
| Locomotion | Move the dog | `unitree_nav2.py` → `/cmd_vel` |
| Arm Control | Move end-effector | `piper_ctrl_single_node.py` → `/pos_cmd` |
| Gripper | Grasp objects | Piper gripper via CAN |

#### Geometry Tools

| Module | Purpose | This Codebase Component |
| --- | --- | --- |
| TF2 Transforms | Coordinate frame math | `tf_pub.py`, ROS2 tf2 |
| Point Cloud Processing | 3D spatial reasoning | Faster-LIO `/cloud_registered` |
| Camera Projection | 2D→3D conversion | `detect_3d_node.py` |

### How Maestro Uses This Codebase

Maestro acts as the "brain" that orchestrates all the ROS components in this SDK:

┌──────────────────────────────────────────────────────────────────┐

│                     MAESTRO (VLM Agent)                          │

│                                                                  │

│  Input: "Find my water bottle and bring it here"                │

│                                                                  │

│  VLM generates Python code that calls:                          │

└──────────────────────────────────────────────────────────────────┘

│                    │                    │

▼                    ▼                    ▼

┌─────────────────┐  ┌─────────────────┐  ┌─────────────────┐

│   PERCEPTION    │  │    PLANNING     │  │    CONTROL      │

│                 │  │                 │  │                 │

│ yolov8_node.py  │  │ Nav2 (ROS2)    │  │ unitree_nav2.py │

│ detect_3d_node  │  │ Faster-LIO     │  │ piper_ctrl_node │

│ RealSense       │  │ Costmaps       │  │ Gripper         │

└────────┬────────┘  └────────┬───────┘  └────────┬────────┘

│                    │                    │

└────────────────────┴────────────────────┘

│

▼

┌─────────────────┐

│   HARDWARE      │

│  Go2W + PiPER   │

└─────────────────┘

### Mapping Maestro Modules to ROS Topics

| Maestro Function | ROS Topic/Service | Node |
| --- | --- | --- |
| `detect_object(class)` | `/yolov8_node/detections` | yolov8_node.py |
| `get_3d_position(det)` | `/detect_3d_node/detections_3d` | detect_3d_node.py |
| `get_camera_image()` | `/camera/color/image_raw` | RealSense |
| `get_depth_image()` | `/camera/depth/image_rect_raw` | RealSense |
| `navigate_to(pose)` | `/goal_pose` | Nav2 |
| `move_base(vx,vy,vyaw)` | `/cmd_vel` | unitree_nav2.py |
| `arm_move_to(x,y,z,r,p,y)` | `/pos_cmd` | piper_ctrl_single_node.py |
| `gripper_close()` | `/pos_cmd` (gripper field) | piper_ctrl_single_node.py |
| `get_robot_pose()` | `/Odometry` | Faster-LIO |
| `get_point_cloud()` | `/cloud_registered` | Faster-LIO |

### VLM Monitoring

Maestro uses a locally-hosted VLM (Qwen2.5-VL-72B-Instruct) running at 2 Hz to:

Monitor task progress via camera feed

Detect unexpected situations

Trigger replanning when needed

### Example Maestro Policy for Assistive Tech

User says: "Get me the medicine bottle from the kitchen counter"

Maestro generates:

# Maestro-generated assistive task policy

# 1. Perception: Find the medicine bottle

detections = call_yolo_detection()

medicine = filter_by_class(detections, "bottle")

if not medicine:

# Active perception: explore to find it

explore_area("kitchen")

detections = call_yolo_detection()

medicine = filter_by_class(detections, "bottle")

# 2. Get 3D position

bottle_3d = get_3d_position(medicine[0])

# 3. Navigate to the bottle

navigate_to(bottle_3d, standoff_distance=0.5)

# 4. Reach and grasp

arm_move_to(bottle_3d.x, bottle_3d.y, bottle_3d.z + 0.1)  # Approach

arm_move_to(bottle_3d.x, bottle_3d.y, bottle_3d.z)        # Lower

gripper_close()

# 5. Retract

arm_move_to(bottle_3d.x, bottle_3d.y, bottle_3d.z + 0.2)

# 6. Return to user

user_position = get_user_position()  # From person detection

navigate_to(user_position)

# 7. Deliver

arm_extend_to_user()

gripper_open()

arm_retract()

print("Medicine delivered!")

### Why Maestro + This Codebase for Assistive Tech

| Benefit | Explanation |
| --- | --- |
| **Zero-shot generalization** | VLM understands novel instructions without retraining |
| **Natural language interface** | Users can give commands in plain English |
| **Closed-loop execution** | Automatically handles failures and replans |
| **Modular architecture** | Easy to add new capabilities (voice, alerts, etc.) |
| **Tested on this hardware** | Maestro was developed specifically for Go2W + PiPER |

### Getting Started with Maestro

**This codebase provides the low-level modules** that Maestro calls

**Maestro provides the high-level reasoning** that interprets user intent

**Together** they enable complex assistive tasks from simple commands

Resources:

Maestro Paper & Code: https://maestro-robot.github.io/

This SDK provides all the ROS modules Maestro needs

__________________________________________________

## Languages Used

| Language | Percentage | Primary Use |
| --- | --- | --- |
| **Python** | ~45% | ROS2 nodes, YOLO detection, arm control, message converters, launch scripts |
| **C++** | ~40% | ROS1 SLAM (Faster-LIO), LiDAR drivers, RealSense drivers, CycloneDDS middleware |
| **XML** | ~8% | ROS package manifests (package.xml), launch files (.launch), URDF robot descriptions |
| **YAML** | ~4% | Configuration files, Nav2 parameters, LiDAR config |
| **Bash/Shell** | ~2% | Environment setup scripts, CAN bus configuration |
| **CMake** | ~1% | Build system configuration |

### Language Breakdown by Component

unitree_ros1/

├── C++ (90%) - Faster-LIO SLAM, HesaiLiDAR driver

├── Python (5%) - Plotting utilities

└── XML/YAML (5%) - Launch files, configs

unitree_ros2/

├── Python (60%) - All ROS2 nodes

├── C++ (25%) - RealSense drivers, CycloneDDS

├── XML (10%) - Package manifests

└── YAML (5%) - Nav2 params, configs

Root/

├── Python (100%) - launch_ros.py, cleanup_ros.py

└── YAML - robot_config.yaml

__________________________________________________

## Directory Structure

unitree_go2w_agent_sdk/

│

├── launch_ros.py              # [Python] Main launcher - starts all components

├── cleanup_ros.py             # [Python] Kills all ROS processes

├── robot_config.yaml          # [YAML] Module enable/disable configuration

├── requirements.txt           # [Text] Python dependencies

│

├── cad_parts/                 # CAD files for custom mounting hardware

│   ├── BOM.csv               # Bill of materials

│   ├── parts/

│   │   ├── png/              # Part images

│   │   ├── steps/            # STEP CAD files (editable)

│   │   └── stl/              # STL files (3D printing)

│   └── png/                  # Assembly images

│

├── images/                    # Documentation images

│   ├── banner.png

│   ├── dog_demo.gif

│   ├── nav2.gif

│   ├── rviz_view.png

│   └── system_overview.png

│

├── unitree_ros1/              # ROS1 Noetic workspace

│   ├── launch/

│   │   └── ros1_system.launch    # [XML] Combined ROS1 launch

│   └── src/

│       ├── faster-lio/           # [C++] LiDAR-Inertial SLAM

│       └── HesaiLidar_ROS_2.0/   # [C++] Hesai LiDAR driver

│

└── unitree_ros2/              # ROS2 Foxy workspace

├── setup.sh                  # [Bash] Environment setup

├── launch/                   # [Python] Launch files

├── cyclonedds_ws/            # [C++] DDS middleware + Unitree messages

└── src/

├── nav2_cloud_bringup/   # [Python] Navigation + dog control

├── yolo_ros/             # [Python] Object detection

├── piper_ros/            # [Python] Robotic arm control

├── realsense-ros/        # [C++] Intel RealSense driver

├── unitree_msg_converter/ # [Python] Message converters

└── rclpy_lifecycle/      # [Python] Lifecycle node utilities

__________________________________________________

## Core System Components

### 1. launch_ros.py (Main Launcher)

Location: ./launch_ros.py

Language: Python

Purpose: Orchestrates the startup of all ROS components in proper sequence

Key Features:

Loads configuration from robot_config.yaml

Manages ROS1 and ROS2 environment sourcing

Launches components with configurable delays

Handles graceful shutdown with process group management

Filters verbose log output

Class: UnitreeRobotLauncher

Methods:

- load_config(config_file)      # Load YAML configuration

- setup_environment()           # Source ROS1/ROS2 environments

- launch_component(name, cmd)   # Start a single component

- launch_all_components()       # Start everything in sequence

- signal_handler()              # Graceful shutdown

- cleanup_all_processes()       # Force kill all ROS processes

Launch Sequence:

ROS1 Core (roscore)

ROS Bridge (ros1_bridge)

RealSense Camera

YOLO Detection (if enabled)

IMU Converter

TF Publishers

Arm Module (if enabled)

LiDAR Driver (if enabled)

SLAM (if enabled)

Navigation (if enabled)

__________________________________________________

### 2. cleanup_ros.py (Process Cleanup)

Location: ./cleanup_ros.py

Language: Python

Purpose: Emergency cleanup script to kill all ROS processes

Processes Killed:

roscore, rosmaster

hesai_ros_driver, faster_lio

ros2, realsense2_camera

robot_state_publisher

imu_converter, relay_joint

ros1_bridge

nav2 components

__________________________________________________

### 3. robot_config.yaml (Configuration)

Location: ./robot_config.yaml

Language: YAML

Purpose: Enable/disable modules and set parameters

Sections:

core_infrastructure:     # ROS1 core + bridge (always needed)

realsense:              # Intel RealSense RGB-D camera

yolo:                   # Object detection settings

arm_module:             # AgileX Piper robotic arm

lidar_driver:           # Hesai LiDAR driver

slam:                   # Faster-LIO SLAM algorithm

navigation:             # Nav2 autonomous navigation

message_converters:     # IMU + TF converters

delays:                 # Startup delay timing

log_filtering:          # Suppress verbose logs

__________________________________________________

## ROS1 Components

### unitree_ros1/src/faster-lio/

Language: C++

Purpose: High-performance LiDAR-Inertial Odometry (SLAM)

Performance: 1000-2000 Hz processing

Key Files:

| File | Purpose |
| --- | --- |
| `src/laser_mapping.cc` | Main SLAM algorithm implementation |
| `src/pointcloud_preprocess.cc` | LiDAR point cloud preprocessing |
| `src/options.cc` | Configuration options parsing |
| `app/run_mapping_online.cc` | Online mapping node |
| `app/run_mapping_offline.cc` | Offline mapping from bag files |

Launch Files:

mapping_hesai.launch - For Hesai LiDAR

mapping_velodyne.launch - For Velodyne LiDAR

mapping_ouster64.launch - For Ouster LiDAR

Published Topics:

/cloud_registered - Registered point cloud

/Odometry - Robot odometry

/path - Trajectory path

__________________________________________________

### unitree_ros1/src/HesaiLidar_ROS_2.0/

Language: C++

Purpose: Driver for Hesai LiDAR sensors

Supported LiDARs:

Pandar40P, Pandar64

PandarXT, PandarQT

AT128, Pandar128

Key Files:

| File | Purpose |
| --- | --- |
| `node/hesai_ros_driver_node.cc` | Main ROS node |
| `config/config.yaml` | LiDAR IP, port, frame settings |

Configuration (config.yaml):

lidar:

- driver:

lidar_type: "PandarXT-32"

msop_port: 2368        # Data port

device_ip: "192.168.1.201"

frame_id: "hesai_lidar"

__________________________________________________

## ROS2 Components

### unitree_ros2/src/nav2_cloud_bringup/

Language: Python

Purpose: Navigation and dog locomotion control

#### unitree_nav2.py - Dog Movement Controller

Location: nav2_cloud_bringup/unitree_nav2.py

Class: CmdVelToUnitree

What it does:

Subscribes to `/cmd_vel` (Twist messages from Nav2)

Converts to Unitree SDK Move() commands

Sends velocity commands to the Go2 robot via subprocess

Parameters:

cmd_vel_topic: '/cmd_vel'      # Input topic

stream_rate_hz: 3.0            # Command frequency

stale_timeout: 0.25            # Stop if no commands

vx_max: 1.2                    # Max forward speed (m/s)

vy_max: 0.5                    # Max lateral speed (m/s)

vyaw_max: 2.5                  # Max rotation speed (rad/s)

How Movement Works:

# Commands are sent via subprocess to avoid SDK conflicts

dog_client = SportClient()

dog_client.Init()

dog_client.Move(vx, vy, vyaw)  # Velocity control

#### move_dog.py - Direct Movement Script

Location: nav2_cloud_bringup/move_dog.py

Purpose: Standalone script for direct dog control

__________________________________________________

### unitree_ros2/src/yolo_ros/

Language: Python

Purpose: YOLOv8/v9 object detection with ROS2 integration

#### yolov8_node.py - 2D Object Detection

Location: yolov8_ros/yolov8_ros/yolov8_node.py

Class: Yolov8Node

What it does:

Loads YOLO model (yolov8m.pt or yolov9c-seg.pt)

Subscribes to camera images

Runs inference on each frame

Publishes detection results

Parameters:

model: "yolov8m.pt"            # Model weights file

device: "cuda:0"               # GPU device (or "cpu")

threshold: 0.5                 # Confidence threshold

enable: True                   # Detection on/off

Detection Output (DetectionArray):

class_id - Object class ID

class_name - Object class name (e.g., "person", "bottle")

score - Confidence score (0-1)

bbox - 2D bounding box (center x, y, width, height)

mask - Segmentation mask (if using seg model)

keypoints - Pose keypoints (if using pose model)

Subscribes:

/camera/color/image_raw - RGB camera images

Publishes:

/yolov8_node/detections - DetectionArray messages

Services:

/yolov8_node/enable - SetBool to toggle detection

__________________________________________________

#### detect_3d_node.py - 3D Object Localization

Location: yolov8_ros/yolov8_ros/detect_3d_node.py

Class: Detect3DNode

What it does:

Receives 2D detections + depth image

Projects 2D bounding boxes to 3D space

Transforms to robot base frame

Publishes 3D bounding boxes

How 3D Projection Works:

# Get depth at bounding box center

z = depth_image[center_y, center_x] / 1000  # mm to meters

# Project to 3D using camera intrinsics

x = z * (center_x - cx) / fx

y = z * (center_y - cy) / fy

# Result: 3D position (x, y, z) in camera frame

Subscribes:

/camera/depth/image_rect_raw - Depth image

/camera/depth/camera_info - Camera calibration

/yolov8_node/detections - 2D detections

Publishes:

/detect_3d_node/detections_3d - DetectionArray with 3D boxes

__________________________________________________

### unitree_ros2/src/piper_ros/

Language: Python

Purpose: AgileX Piper 6-DOF robotic arm control

#### piper_ctrl_single_node.py - Arm Controller

Location: piper/piper/piper_ctrl_single_node.py

Class: PiperRosNode

What it does:

Connects to Piper arm via CAN bus

Publishes joint states at 200 Hz

Accepts end-effector pose commands

Controls gripper

Parameters:

can_port: 'can0'               # CAN interface name

auto_enable: False             # Auto-enable motors on start

gripper_exist: True            # Gripper attached

rviz_ctrl_flag: False          # RViz control mode

Published Topics:

/joint_states_single - 7 joint angles (6 arm + 1 gripper)

/arm_status - Motor status, errors, limits

/end_pose - End-effector pose (position + quaternion)

Subscribed Topics:

/pos_cmd - End-effector pose command (PosCmd message)

/enable_flag - Enable/disable arm motors

Services:

/piper/enable_srv - Enable/disable motors with timeout

Arm Control Methods:

# Joint control (angles in degrees * 1000)

piper.JointCtrl(j0, j1, j2, j3, j4, j5)

# End-effector pose control (mm * 1000, degrees * 1000)

piper.EndPoseCtrl(x, y, z, rx, ry, rz)

# Gripper control

piper.GripperCtrl(angle, force, mode, extra)

# angle: 0-80000 (closed to open)

# force: grip force

# mode: 0x01=position, 0x02=force

__________________________________________________

#### Shell Scripts for CAN Bus

can_activate.sh:

# Activate CAN interface

sudo ip link set can0 type can bitrate 1000000

sudo ip link set up can0

find_all_can_port.sh:

# Auto-detect CAN ports

ip link show | grep can

__________________________________________________

### unitree_ros2/src/unitree_msg_converter/

Language: Python

Purpose: Convert between Unitree proprietary messages and standard ROS messages

#### imu_converter.py - IMU Message Converter

Location: unitree_msg_converter/imu_converter.py

Class: LowstateToImu

What it does:

Subscribes to Unitree `/lowstate` messages

Extracts IMU data (quaternion, gyroscope, accelerometer)

Publishes standard `sensor_msgs/Imu` messages

Subscribes:

/lowstate - Unitree LowState (proprietary)

Publishes:

/imu/data - Standard IMU message

Data Conversion:

# Unitree LowState contains:

# - imu_state.quaternion[4]    -> orientation

# - imu_state.gyroscope[3]     -> angular_velocity

# - imu_state.accelerometer[3] -> linear_acceleration

__________________________________________________

#### relay_joint.py - Joint State Relay

Purpose: Maps gripper angles to prismatic joint goals for URDF visualization

#### tf_pub.py - TF Publisher

Purpose: Publishes static transforms between coordinate frames

__________________________________________________

### unitree_ros2/src/realsense-ros/

Language: C++

Purpose: Intel RealSense RGB-D camera driver

Key Files:

| File | Purpose |
| --- | --- |
| `src/base_realsense_node.cpp` | Main camera node |
| `src/ros_sensor.cpp` | Sensor abstraction |
| `launch/rs_launch.py` | Launch configuration |

Published Topics:

/camera/color/image_raw - RGB image (640x480)

/camera/depth/image_rect_raw - Depth image

/camera/depth/camera_info - Camera calibration

/camera/aligned_depth_to_color/image_raw - Aligned depth

Launch Parameters:

depth_module.profile: '640x480x6'    # Resolution @ 6fps

rgb_camera.profile: '640x480x6'

enable_sync: True

pointcloud.enable: True

__________________________________________________

### unitree_ros2/cyclonedds_ws/

Language: C++

Purpose: DDS middleware and Unitree message definitions

Key Components:

cyclonedds/ - CycloneDDS implementation

rmw_cyclonedds/ - ROS2 middleware wrapper

unitree/ - Unitree message types:

unitree_api/ - Request/Response messages

unitree_go/ - Go2 robot messages (LowState, etc.)

unitree_hg/ - H1 humanoid messages

__________________________________________________

## Configuration Files

### Nav2 Parameters

Location: unitree_ros2/src/nav2_cloud_bringup/launch/nav2_params_online_fasterlio.yaml

Key Parameters:

controller_server:

FollowPath:

plugin: "dwb_core::DWBLocalPlanner"

max_vel_x: 0.5          # Max forward speed

max_vel_theta: 0.5      # Max rotation speed

acc_lim_x: 2.5          # Acceleration limit

local_costmap:

obstacle_layer:

observation_sources: scan

scan:

topic: /cloud_registered

max_obstacle_height: 2.0

clearing: True

__________________________________________________

## Key Python Files Reference

| File | Location | Purpose |
| --- | --- | --- |
| `launch_ros.py` | Root | Main system launcher |
| `cleanup_ros.py` | Root | Process cleanup utility |
| `unitree_nav2.py` | nav2_cloud_bringup | Dog velocity control |
| `move_dog.py` | nav2_cloud_bringup | Direct dog movement |
| `yolov8_node.py` | yolov8_ros | 2D object detection |
| `detect_3d_node.py` | yolov8_ros | 3D object localization |
| `tracking_node.py` | yolov8_ros | Object tracking |
| `debug_node.py` | yolov8_ros | Detection visualization |
| `piper_ctrl_single_node.py` | piper | Arm control node |
| `imu_converter.py` | unitree_msg_converter | IMU message conversion |
| `tf_pub.py` | unitree_msg_converter | TF publishing |
| `relay_joint.py` | unitree_msg_converter | Joint state relay |

__________________________________________________

## ROS Topics & Services

### Control Topics

| Topic | Type | Direction | Purpose |
| --- | --- | --- | --- |
| `/cmd_vel` | geometry_msgs/Twist | Subscribe | Dog velocity commands |
| `/goal_pose` | geometry_msgs/PoseStamped | Subscribe | Navigation goal |
| `/pos_cmd` | piper_msgs/PosCmd | Subscribe | Arm end-effector command |
| `/enable_flag` | std_msgs/Bool | Subscribe | Arm enable/disable |

### State Topics

| Topic | Type | Direction | Purpose |
| --- | --- | --- | --- |
| `/lowstate` | unitree_go/LowState | Publish | Dog state (IMU, motors) |
| `/imu/data` | sensor_msgs/Imu | Publish | Converted IMU data |
| `/joint_states_single` | sensor_msgs/JointState | Publish | Arm joint angles |
| `/end_pose` | geometry_msgs/Pose | Publish | Arm end-effector pose |
| `/arm_status` | piper_msgs/PiperStatusMsg | Publish | Arm status/errors |

### Perception Topics

| Topic | Type | Direction | Purpose |
| --- | --- | --- | --- |
| `/camera/color/image_raw` | sensor_msgs/Image | Publish | RGB camera |
| `/camera/depth/image_rect_raw` | sensor_msgs/Image | Publish | Depth image |
| `/yolov8_node/detections` | yolov8_msgs/DetectionArray | Publish | 2D detections |
| `/detect_3d_node/detections_3d` | yolov8_msgs/DetectionArray | Publish | 3D detections |
| `/cloud_registered` | sensor_msgs/PointCloud2 | Publish | LiDAR point cloud |

### Services

| Service | Type | Purpose |
| --- | --- | --- |
| `/piper/enable_srv` | piper_msgs/Enable | Enable/disable arm |
| `/yolov8_node/enable` | std_srvs/SetBool | Enable/disable detection |

__________________________________________________

## Hardware Interfaces

### Unitree Go2W Dog

Connection: Ethernet (eth0)

IP: Configured via DHCP or static

SDK: unitree_sdk2py

from unitree_sdk2py.go2.sport.sport_client import SportClient

from unitree_sdk2py.core.channel import ChannelFactoryInitialize

ChannelFactoryInitialize(0, "eth0")

dog = SportClient()

dog.Init()

dog.Move(vx, vy, vyaw)  # Velocity control

### AgileX Piper Arm

Connection: CAN Bus (can0)

Baud Rate: 1,000,000 bps

SDK: piper-sdk

from piper_sdk import C_PiperInterface

arm = C_PiperInterface(can_name="can0")

arm.ConnectPort()

arm.EnableArm(7)  # Enable all 7 axes

arm.JointCtrl(j0, j1, j2, j3, j4, j5)  # Joint angles

arm.EndPoseCtrl(x, y, z, rx, ry, rz)   # Cartesian

arm.GripperCtrl(angle, force, mode, 0)  # Gripper

### Intel RealSense

Connection: USB 3.0

Models Supported: D400 series, T265

### Hesai LiDAR

Connection: Ethernet

Default IP: 192.168.1.201

Data Port: 2368

__________________________________________________

## Build & Run Commands

### Initial Setup

# Install ROS dependencies

sudo apt install ros-foxy-nav2-* ros-foxy-cv-bridge ros-foxy-tf2-ros

# Install Python dependencies

pip3 install piper-sdk>=0.6.0 python-can>=4.0.0 ultralytics

# Build ROS1 workspace

cd unitree_ros1

catkin_make

# Build ROS2 workspace

cd unitree_ros2

colcon build

### Running the System

# Full system launch

python3 launch_ros.py robot_config.yaml

# With custom config

python3 launch_ros.py my_config.yaml

# Cleanup after crash

python3 launch_ros.py --cleanup

# or

python3 cleanup_ros.py

### CAN Bus Setup (for arm)

# Activate CAN interface

sudo ip link set can0 type can bitrate 1000000

sudo ip link set up can0

# Verify

ip link show can0

__________________________________________________

## Extension Points for Assistive Tech

### Option A: Use Maestro (Recommended for Complex Tasks)

For sophisticated assistive tasks that require natural language understanding, planning, and error recovery, use Maestro as your high-level controller:

┌────────────────────────────────────────────────────────┐

│                   YOUR APPLICATION                      │

│                                                        │

│   User Interface (Voice/App/Button)                    │

│              │                                         │

│              ▼                                         │

│   ┌─────────────────────────────────┐                 │

│   │         MAESTRO                  │                 │

│   │   - Interprets user requests     │                 │

│   │   - Plans multi-step tasks       │                 │

│   │   - Handles failures/replanning  │                 │

│   │   - Uses VLM for reasoning       │                 │

│   └─────────────────────────────────┘                 │

│              │                                         │

│              ▼                                         │

│   This Codebase (ROS modules)                         │

└────────────────────────────────────────────────────────┘

When to use Maestro:

Tasks requiring multi-step planning ("get X from Y and bring it here")

Tasks needing visual reasoning ("find the red object")

Tasks that may fail and need replanning

Natural language interfaces

Setup:

Run this codebase's ROS system: `python3 launch_ros.py robot_config.yaml`

Run Maestro with connection to the ROS topics

Maestro generates Python code that calls ROS topics/services

### Option B: Write Custom ROS Nodes (Simpler Tasks)

For simpler, well-defined tasks, write custom ROS2 nodes directly:

### 1. Create a New Assistive Task Node

#!/usr/bin/env python3

import rclpy

from rclpy.node import Node

from yolov8_msgs.msg import DetectionArray

from geometry_msgs.msg import PoseStamped, Twist

from piper_msgs.msg import PosCmd

class AssistiveTaskNode(Node):

def __init__(self):

super().__init__('assistive_task')

# Subscribe to detections

self.detection_sub = self.create_subscription(

DetectionArray,

'/yolov8_node/detections',

self.detection_callback,

10

)

# Publishers for control

self.cmd_vel_pub = self.create_publisher(Twist, '/cmd_vel', 10)

self.goal_pub = self.create_publisher(PoseStamped, '/goal_pose', 10)

self.arm_pub = self.create_publisher(PosCmd, '/pos_cmd', 10)

def detection_callback(self, msg):

for detection in msg.detections:

if detection.class_name == "bottle":

self.get_logger().info(f"Found bottle at confidence {detection.score}")

# Navigate to object, pick it up, etc.

def main():

rclpy.init()

node = AssistiveTaskNode()

rclpy.spin(node)

rclpy.shutdown()

if __name__ == '__main__':

main()

### 2. Add Voice Commands

# Install: pip3 install vosk sounddevice

import vosk

import sounddevice as sd

class VoiceCommandNode(Node):

def __init__(self):

super().__init__('voice_command')

self.model = vosk.Model("model")

self.recognizer = vosk.KaldiRecognizer(self.model, 16000)

def process_audio(self, audio_data):

if self.recognizer.AcceptWaveform(audio_data):

result = self.recognizer.Result()

text = json.loads(result)["text"]

if "fetch" in text and "water" in text:

self.fetch_object("bottle")

elif "come here" in text:

self.navigate_to_user()

### 3. Person Following Mode

class PersonFollowerNode(Node):

def __init__(self):

super().__init__('person_follower')

self.create_subscription(

DetectionArray,

'/detect_3d_node/detections_3d',

self.follow_person,

10

)

self.cmd_pub = self.create_publisher(Twist, '/cmd_vel', 10)

self.target_distance = 1.5  # meters

def follow_person(self, msg):

for det in msg.detections:

if det.class_name == "person":

# Get 3D position of person

x = det.bbox3d.center.position.x

z = det.bbox3d.center.position.z

# Calculate velocity to maintain distance

distance = math.sqrt(x**2 + z**2)

error = distance - self.target_distance

cmd = Twist()

cmd.linear.x = 0.3 * error  # Proportional control

cmd.angular.z = -0.5 * math.atan2(x, z)  # Turn towards person

self.cmd_pub.publish(cmd)

### 4. Object Retrieval Pipeline

1. User requests object (voice/app/button)

2. YOLO detects object in camera view

3. 3D node localizes object position

4. Nav2 navigates robot to object

5. Piper arm picks up object

6. Robot returns to user

7. Arm delivers object

### 5. Custom YOLO Model Training

Train for assistive-specific objects:

Wheelchairs, walkers, canes

Medication bottles

Common household items

User's personal items

# Train custom model

yolo train model=yolov8n.pt data=assistive_objects.yaml epochs=100

__________________________________________________

## Troubleshooting

### Common Issues

1. "Cannot connect to rosmaster"

ROS1 core not running

Run: roscore in separate terminal

2. "CAN interface not found"

CAN not activated

Run: sudo ip link set up can0

3. "No images from RealSense"

USB bandwidth issue

Try different USB port or lower resolution

4. "YOLO out of memory"

Set device: "cpu" in robot_config.yaml

Or reduce image resolution

5. "Nav2 not responding"

Check if SLAM is providing odometry

Verify /cloud_registered topic is publishing

### Debug Commands

# Check ROS2 topics

ros2 topic list

ros2 topic echo /yolov8_node/detections

# Check transforms

ros2 run tf2_tools view_frames

# View camera

ros2 run rqt_image_view rqt_image_view

# Monitor CAN bus

candump can0

__________________________________________________

## Contact & Resources

### Primary Resources

Maestro (VLM Robot Control): https://maestro-robot.github.io/

Unitree SDK Documentation: https://github.com/unitreerobotics

Piper SDK: https://github.com/agilexrobotics/piper_sdk

### Supporting Technologies

Nav2 Documentation: https://navigation.ros.org/

YOLOv8 Documentation: https://docs.ultralytics.com/

ROS2 Foxy: https://docs.ros.org/en/foxy/

Intel RealSense: https://github.com/IntelRealSense/realsense-ros

### The Full Stack

┌─────────────────────────────────────────────────────────────────┐

│  YOUR ASSISTIVE TECH APPLICATION                                │

│  (Voice interface, mobile app, accessibility controls)          │

└─────────────────────────────────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────────────────────────────┐

│  MAESTRO - VLM Policy Planner                                   │

│  https://maestro-robot.github.io/                               │

│  - Natural language understanding                               │

│  - Task planning & code generation                              │

│  - Closed-loop execution with replanning                        │

└─────────────────────────────────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────────────────────────────┐

│  THIS CODEBASE - unitree_go2w_agent_sdk                         │

│  - ROS1/ROS2 bridge infrastructure                              │

│  - Perception modules (YOLO, 3D detection, RealSense)           │

│  - Planning modules (Nav2, Faster-LIO SLAM)                     │

│  - Control modules (dog movement, arm control)                  │

└─────────────────────────────────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────────────────────────────┐

│  HARDWARE                                                       │

│  - Unitree Go2W quadruped robot                                 │

│  - AgileX Piper 6-DOF arm + gripper                            │

│  - Intel RealSense RGB-D camera                                 │

│  - Hesai LiDAR                                                  │

└─────────────────────────────────────────────────────────────────┘

__________________________________________________

*This documentation is for internal BCIT Make+ use. Last updated January 2025.*